BridgeTrafficLoadSim:

Long Run Simulation Model

for Bridge Loading

Version 1.3.5

User Manual

Dr Colin Caprani

Monash University

Acknowledgements

This program is based on three separate programs developed as part of the author’s PhD research from 2001. In turn, these were based on work by Dr Samuel Grave, former PhD student at Trinity College Dublin. The current program which encompasses the functions of two of the previous programs has evolved since 2007 and has been much influenced by the work of Dr Bernard Enright, DIT.

For further information, please contact colin.caprani@monash.

Dr Colin Caprani,

July 2012

Contents

1.	Introduction	5

1.1	The BridgeTrafficLoadSim Program	5

1.2	The User Manual	6

1.3	Program Release History	8

2.	About BTLS	12

2.1	Introduction	12

2.2	BTLS: Capabilities and Limitations	13

3.	BTLS Input	15

3.1	Introduction	15

3.2	Traffic Files	17

3.3	Configuration File	31

3.4	Bridge Definition File	43

3.5	Lane Flow Data File	47

3.6	Influence Line Definition File	50

3.7	Influence Surface Definition File	54

4.	Using BTLS	57

4.1	Running the program	57

4.2	Console output	58

4.3	Input Errors	61

5.	BTLS Output	62

5.1	Introduction	62

5.2	Miscellaneous Output	63

5.3	Vehicle Output	67

5.4	Block Maximum Files	69

5.5	Peaks-Over-Threshold Files	72

5.6	Load Effect Statistics Output	75

5.7	Loading Event File Structure	77

6.	Appendices	79

6.1	Appendix 1 – Traffic File Formats	79

6.2	Appendix 2 – References	83

# Introduction

The BridgeTrafficLoadSim Program

BridgeTrafficLoadSim will be referred to as BTLS hereafter.

BTLS generates artificial traffic and passes it across bridges determining various load effects. The traffic is generated according to a relatively simple model. There are several built-in influence lines for various load effects, but the user can input their own influence line also. The program outputs various quantities of interest, which are controllable by the user.

These programs have been in use in DIT and University College Dublin over a 10-year period. There have been multiple users, and so exhibit a fair degree of maturity. That is, the user should not get unexpected results when used correctly. Whilst the routines have been thoroughly tested many times, with ongoing changes, it is good practice to satisfy oneself as to the accuracy of the programs. Various forms of output should assist with this process.

## The User Manual

### Purpose

This User Manual has been written to explain the use of the BTLS program, and to explain its capabilities and limitations.

Notices

Points of significant importance are denoted as:

Important!

Typically, failure to adhere to these points will result in unexpected behaviour or a program crash.

### Glossary

Load Effect	The result of a calculation using any influence line. Total load on the bridge is sometimes referred to as a load effect therefore.

## Program Release History

| Version | Date | Description |
| --- | --- | --- |
| 1.0.0 | 5/7/12 | - Initial release to international users |
| 1.0.1 | 21/7/12 | - Added version number on screen output - Fixed problem with AllEvents output – it now outputs the last unfilled buffer properly. |
| 1.0.2 | 24/7/12 | - Added a FatigueEvents output file type with max and min values of loading events in it. Only output if AllEvents is output - temporarily |
| 1.0.3 | 27/7/12 | - Bug fix: truck departures not always correctly calculated - fixed using 1e300 for timeOff variables. - Bug fix: reading single lane vehicle files caused crash - fixed. - Minor console output changes for more user information |
| 1.0.4 | 13/8/12 | - Console output for missing files. - Bug fix: reading multiple lane vehicle files crashed following v1.0.3 fix for single lanes! |
| 1.0.5 | 27/8/12 | - Separate discrete influence lines now possible for each lane (IL option 2 in bridge definition file). |
| 1.1.0 | 27/10/12 | - Peaks over Threshold output - Basic statistics output - Created inheritance structure for output types - Flow Data statistics output - Restructured the BTLSin file - Fixed some bugs, especially one on flow generation - Renamed output files for consistency |
| 1.2.0 | 28/11/12 | - Added Influence Surface (IS) calculations - Amended BTLSin file structure accordingly - Added new file structure – DITIS for ISs, including wheel track width for each axle - Added POT counter file output and input specs in BTLSin - Program can now run without IL or IS files being present in folder, once not required. - Added option for vehicle output file format - Traffic folder location now can be specified in BTLSin - Updated generation of tri-modal-normal distributions to include deterministic and single-generation value for axle track widths - Added transverse position in lane variability through BTLSin - Added transverse axle track width generation file (ATW.csv) input and default value options |
| 1.2.1 | 20/12/12 | - Bug fix: influence surface calculations did not take transverse position into account – now fixed. - Bug fix: crash caused by traffic files with no vehicles in a lane is fixed. |
| 1.2.2 | 27/3/2014 | - Bug fix: VehicleBuffer.h(31) now max axles 20 - Improved feedback on incorrect load effect definitions |
| 1.2.3 | 14/6/2019 | - TH file time precision is now 3 places - Bridge/IS files now can include comment lines ‘\\’ |
| 1.2.4 | 12/8/2019 | - Added number of events of each truck count to statistics output |
| 1.2.5 | 21/08/19 | - Added MON file format - Cleaned up all warnings - Corrected minor bugs in flow/sim time calculations |
| 1.2.6 | 25/08/19 | - Generic classification of vehicles - Improved warnings - Clarified local and global lane numberings - Block numbering fixed to relative sim time - Correct MON file bug |
| 1.3.0 | 28/08/19 | - Generic flow and vehicle generation routines - Flow and vehicle models can now be mixed - Added new garage vehicle generator |
| 1.3.1 | 30/08/19 | - Minor bug fixes (kernels & RNG seed) - Started OpenMP, but crashes in x64 release - Bug fix: RNG seed, and kernels - Converted to shared_ptr - Still has memory leaks in CConfigData |
| 1.3.2 | 2/09/19 | - Fixed CVehicleBuffer bug, storing every vehicle forever. - Moved input files to Working directory - clarified memory leak reports - improved smart pointer useage |
| 1.3.3 | 2/09/19 | - Corrected overlap bug, and improved reporting of potential overlaps |
| 1.3.4 | 3/09/19 | - Fixed bug in MinGap calculations - Necessary to re-order sequence of start up |
| 1.3.5 | 17/09/19 | - RNG updated to C++11 STL - Updated Project Configurations: changed exe to x64 default, and now x86 is named x86 - More warnings quashed - Minor performance tweaks |

### Installing the Program

BTLS does not require installation, it is a standalone executable program. It has been tested on Windows XP, Vista, and Windows 7.

BTLS can be run in a 64 bit version, which is more efficient that the 32 bit version. The program is a single- threaded application and so cannot take advantage of multi-core processors. Therefore for maximum speed, prefer a computer with a fast single processor over a computer with a multi-core slower processor.

The program does not require much memory because:

- Only a small amount of input information is held in memory;

- Traffic is generated in 1-day blocks on a rolling basis;

- Output to file is made according to user input: this balances accessing eh hard drive (which is slow) and memory requirements. Prefer to use memory than output to the hard drive often.

A computer with 1 GB of RAM is sufficient and other programs can continue to operate successfully.

BTLS uses two folders to operate:

- Working folder: the current folder in which configuration files and the executable exist;

- Traffic folder: a folder on the computer in which the traffic characteristics for sites are stored.

# About BTLS

Introduction

BTLS performs efficient calculations of static traffic actions on bridges. It can generate artificial traffic and write it to file. It can calculate load effects from traffic files. However such simulations are limited by the file size that can be held in the computer memory. In an alternate mode, it can generate traffic and determine load effects simultaneously without recourse outputting traffic to file. In this mode the program can simulate 100s of years of load effect data quite quickly. The exact speed depends on many parameters, but in the worst case, 1000 years has been simulated in under 20 hours. For more typical cases 100 years takes about 1 hour to simulate.

BTLS is provided in two versions:

- BridgeTrafficLoadSim.exe: the 32-bit version.

- BridgeTrafficLoadSim_x64.exe: the 64-bit version, found to run faster than 32-bit version on 64-bit machines.

## BTLS: Capabilities and Limitations

### Capabilities

BTLS is able to:

- Generate artificial traffic from the traffic model of Caprani (2005 & 2012).

- Read in traffic and pass it over influence lines and surfaces;

- Use lane factors to account for lateral distribution of load effect due to transverse stiffness of the bridge;

- Use user-defined influence lines;

- Use separate influence lines for each lane of traffic (user-defined or in-built);

- Determine static load effects from generated or read-in traffic passing over defined bridges and either user-defined or built-in influence lines;

- Model one or two directions at the same time, with any number of lanes in each direction;

- Output different types of data for debugging and further analysis, as specified by the user.

- Output a file suitable for further fatigue analysis.

- Output data for block maxima or peaks over threshold approaches.

- Output traffic flow statistics and load effect statistics

Future plans include:

- Built-in fatigue calculation;

- Improved traffic model for greater generality;

- Possibly a visual user input interface.

### Limitations

BTLS is not able to:

- Generate trucks with more than 5 axles;

- Determine the number of lanes or directions of traffic in the specified input traffic file, in advance of a simulation;

- Determine the input traffic file format;

- Determine dynamic load effects;

- Perform extrapolations for return periods.

Note that cars are assumed to be 4 m long and have GVW of 2 tonnes evenly distributed to each axle.

# BTLS Input

Introduction

BTLS operates in one of three modes, which are numbered:

- Gen & Sim: In this mode traffic is generated in the program and simulated crossing the defined bridges;

- Gen: In this mode traffic is generated and output to file;

- Read & Sim: In this mode traffic is read from a file and simulated crossing the defined bridges.

Different types of input are required:

- Traffic model files: for the generation of artificial random traffic;

- Configuration file: the main user input file which configures each run of the program;

- Supporting files: define bridges, influence lines and traffic flows to be used in the simulation.

Thus for a successful run the files required are:

| Location | Files |
| --- | --- |
| Traffic folder defined in BTLSin.txt, usually C:\Traffic\[The Site] | Several – see Traffic Data input files section |
| Working Folder (anywhere on computer) | BridgeTrafficLoadSim.exe (or 64 bit version) BTLSin.txt Lane flow definition file Bridge definition file Influence line definition file Influence Surface definition file |

Depending on the program mode and input values, BTLS will run if some of the above files are missing or not accessible. Warnings are usually given.

## Traffic Files

### The Grave Traffic Model (Model 0)

The model describing the physical characteristics of the traffic is defined in a series of files located in a folder, named after the site which is located, which is a sub-folder to the Traffi Folder. As of v1.2.0 this location can be specified in BTLSin.txt, but it is recommended to keep it at C:\Traffic\.

Important!

The traffic folder must reside at the location specified in BTLSin.txt (usually C:\Traffic\).

The traffic model is described by Caprani (2005), and is based on Grave (2001). Presently, 13 sites have been modelled accordingly and are indexed by BTLS as follows, and are located in sub-folders of the site name below:

| Site Indices |  |
| --- | --- |
| Index | Site |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 | Angers Auxerre A196 B224 A296 SAMARIS\D1 SAMARIS\D2 SAMARIS\D3 SAMARIS\S1 SAMARIS\S2 SAMARIS\S3 SAMARIS\D SAMARIS\S |

Auxerre is particularly important as the Eurocode load model LM1 was initially calibrated upon this traffic.

### Traffic Data Input Files

The files then placed in this folder are of type comma separated values (*.csv). These file types are easily created in a spread sheet program, but can also be read or edited in a text editor.

Many of the vehicles properties are modelled with a three-mode normal distribution; that is, the data may be multi-modally normally distributed. There are three parameters required for each of the modes: the weight,  the mean,  and the standard deviation,  The maximum number of modes allowed for is three; hence the 3×3 tabular format of the data. The units of the data are as per the traffic file convention explained in the Output section.

Important!

The files names must be as given for each modelled property.

The input defining the traffic flow and composition is made in the working folder as the executable.

Important!

The current traffic model only accounts for vehicles with up to 5 axles.

Note:

In the following, tri-modal normal distributions are often used to model parameters. Should deterministic values be needed instead, the distribution should have only one mode, set the mean to the required value, and set the standard deviation to zero.

#### Axle Spacing Definition

Asall.csv

This file stores the axle spacing data for all classes of trucks measured at the site. The values must be separated by commas. An example is:

1,50.7,3.7,0,0,0,0,0,0,0,0,0

0,0,0,0,0,0,0,0,0,0,0,0

0,0,0,0,0,0,0,0,0,0,0,0

0.65,34.1,6.9,1,11.5,1.7,0,0,0,0,0,0

0.268,34,1.5,0,0,0,0,0,0,0,0,0

0.082,61.5,6,0,0,0,0,0,0,0,0,0

0.672,30.6,1.5,0.153,34.7,3,0.317,11.8,0.6,0,0,0

0.328,30.2,3.9,0.386,54.8,8.6,0.598,12.1,1.7,0,0,0

0,0,0,0.461,59.5,3.4,0.085,18.3,0.9,0,0,0

0.041,23.2,1.4,0.133,42,5.6,1,10.9,1.7,1,11,1.7

0.959,30.4,1.8,0.867,51.2,3.4,0,0,0,0,0,0

0,0,0,0,0,0,0,0,0,0,0,0

This data may be more easily understood viewed in tabular form. The meaning of the rows and columns is also shown in relation to the ti-mode normal distribution adopted.

| Class | Line | Spacing 1-2 |  |  | Spacing 2-3 |  |  | Spacing 3-4 |  |  | Spacing 4-5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-Axle | 1 | 1 | 50.7 | 3.7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3-Axle | 5 | 0.65 | 34.1 | 6.9 | 1 | 11.5 | 1.7 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 6 | 0.268 | 34 | 1.5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 7 | 0.082 | 61.5 | 6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4-Axle | 9 | 0.672 | 30.6 | 1.5 | 0.153 | 34.7 | 3 | 0.317 | 11.8 | 0.6 | 0 | 0 | 0 |
|  | 10 | 0.328 | 30.2 | 3.9 | 0.386 | 54.8 | 8.6 | 0.598 | 12.1 | 1.7 | 0 | 0 | 0 |
|  | 11 | 0 | 0 | 0 | 0.461 | 59.5 | 3.4 | 0.085 | 18.3 | 0.9 | 0 | 0 | 0 |
|  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5-Axle | 13 | 0.041 | 23.2 | 1.4 | 0.133 | 42 | 5.6 | 1 | 10.9 | 1.7 | 1 | 11 | 1.7 |
|  | 14 | 0.959 | 30.4 | 1.8 | 0.867 | 51.2 | 3.4 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 15 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

#### Axle Track Width

ATW.csv

This file is required when generating vehicles for use with an influence surface. It specifies the width from centre of pressure of the wheels on one side of the axle, to the other. In general this dimension is different to that of vehicle width, since the tyre width and number of tyres must be accounted for. This file is optional, and if it is missing then a default value specified in BTLSin.txt will be used instead.

This file again uses tri-modal normal distributions to model the track width of each axle for each class of vehicle. The distributions are specified using 3×3 matrices as is done for Axle Spacings. The file structure is then:

| Class | Line Nos. | Axle 1 | Axle 2 | Axle 3 | Axle 4 | Axle 5 |
| --- | --- | --- | --- | --- | --- | --- |
| 2-axle | 1-3 | 3×3 | 3×3 | N/A | N/A | N/A |
|  | 4 |  |  |  |  |  |
| 3-axle | 5-7 | 3×3 | 3×3 | 3×3 | N/A | N/A |
|  | 8 |  |  |  |  |  |
| 4-axle | 9-11 | 3×3 | 3×3 | 3×3 | 3×3 | N/A |
|  | 12 |  |  |  |  |  |
| 5-axle | 13-15 | 3×3 | 3×3 | 3×3 | 3×3 | 3×3 |

An example file is shown below which illustrates three separate model types for the track width:

- A typical implementation using tri-modal distributions for each axle track width is shown for the 2-axle truck type. Notice the zero values for the redundant axles 3, 4, and 5.

- Single values of track width can be specified for all the axles by only defining a distribution for the first axle, and leaving subsequent values all zero. This is done for 3-axle trucks in the example below.

- Deterministic values can be defined as noted previously by specifying only one model, with mean of the require value and standard deviation of zero. This is done in the example file below for 4- and 5-axle trucks, showing the different track widths for the different axle types (steering, drive, trailer with dual tyres).

The track widths are specified in units of centimetre.

0.109,188,27.3,0.109,188,27.3,0,0,0,0,0,0,0,0,0

0.847,193.9,6.4,0.847,193.9,6.4,0,0,0,0,0,0,0,0,0

0.045,167.6,6.1,0.045,167.6,6.1,0,0,0,0,0,0,0,0,0

0.109,188,27.3,0,0,0,0,0,0,0,0,0,0,0,0

0.847,193.9,6.4,0,0,0,0,0,0,0,0,0,0,0,0

0.045,167.6,6.1,0,0,0,0,0,0,0,0,0,0,0,0

1,196,0,1,184,0,1,199,0,1,199,0,0,0,0

0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

1,196,0,1,184,0,1,199,0,1,199,0,1,199,0

0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

#### Axle Weights

Aw2&3.csv

Two files are used, one for 2 and 3 axle trucks, the other for 4 and 5 axle trucks. This file contains the axle weight information for the 2- and 3- axle trucks of the site. An example is:

0.560,33.4,3.7,0.440,59.4,7.4,0.000,0.0,0.0

0.440,40.6,7.4,0.560,66.6,3.7,0.000,0.0,0.0

0.000,0.0,0.0,0.000,0.0,0.0,0.000,0.0,0.0

0.066,20.4,1.5,0.769,34.6,6.8,0.558,30.5,5.9

0.522,26.0,4.9,0.227,39.2,2.2,0.442,37.7,3.5

0.412,38.7,8.6,0.004,54.4,3.7,0.000,0.0,0.0

This data is explained as follows:

| Class | Row | Weight Axle 1 |  |  | Weight Axle 2 |  |  | Weight Axle 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-Axle | 1 | 0.56 | 33.4 | 3.7 | 0.44 | 59.4 | 7.4 | 0 | 0 | 0 |
|  | 2 | 0.44 | 40.6 | 7.4 | 0.56 | 66.6 | 3.7 | 0 | 0 | 0 |
|  | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 4 |  |  |  |  |  |  |  |  |  |
| 3-Axle | 5 | 0.066 | 20.4 | 1.5 | 0.769 | 34.6 | 6.8 | 0.558 | 30.5 | 5.9 |
|  | 6 | 0.522 | 26 | 4.9 | 0.227 | 39.2 | 2.2 | 0.442 | 37.7 | 3.5 |
|  | 7 | 0.412 | 38.7 | 8.6 | 0.004 | 54.4 | 3.7 | 0 | 0 | 0 |

Aw4&5.csv

This file contains the axle weight information for the 4- and 5-axle trucks. It has been found that the axle weights of the 4- and 5-axle trucks depend on the Gross Vehicle Weight (GVW). Thus the data governing these axle weights have been assembled for 12 classes of truck GVW, beginning at 25 kN and increasing in steps of 50 kN.

0.0,0.0,0.0,0.0,0.0,0.0

20.9,39.8,39.3,5.2,6.9,7.3

25.6,36.5,38.0,5.4,4.8,5.7

23.9,35.5,40.7,4.3,4.6,5.2

20.3,36.1,43.6,3.6,4.6,5.4

17.4,34.9,47.7,3.0,4.1,5.5

14.8,33.4,51.8,2.1,3.1,4.1

14.5,33.6,51.9,1.5,2.6,3.2

13.9,32.4,53.7,1.3,2.3,3.1

11.9,31.4,56.7,0.9,1.4,0.9

0.0,0.0,0.0,0.0,0.0,0.0

0.0,0.0,0.0,0.0,0.0,0.0

0.0,0.0,0.0,0.0,0.0,0.0

0.0,0.0,0.0,0.0,0.0,0.0

19.1,36.5,44.5,6.0,7.4,7.2

23.6,32.8,43.7,4.6,4.2,5.0

21.4,33.4,45.3,3.2,4.8,5.4

18.1,33.8,48.1,2.4,4.5,5.5

15.7,32.3,52.0,1.8,3.8,4.7

14.3,31.0,54.6,1.5,3.3,3.9

13.4,29.6,57.1,1.2,2.9,3.4

12.7,27.7,59.6,1.0,2.7,3.1

0.0,0.0,0.0,0.0,0.0,0.0

0.0,0.0,0.0,0.0,0.0,0.0

A single line separates the 4- and 5-axle data. The six entries for each line, or GVW range of truck, represent the parameters of the single-mode Normal distributions for the first (W1) and second (W2) axles and the total weight of the tandem or tridem (WT) in the following order:

| Mean W1 | Mean W2 | Mean WT | SD W1 | SD W2 | SD WT |
| --- | --- | --- | --- | --- | --- |

This has resulted from previous research which has found that the weights of the axles in the tandem or tridem of 4- and 5-axle trucks (respectively) are equal and thus the tandem/tridem may be considered as one weight. The calculated tandem/tridem weight are divided by the number of axles to give each axle a weight in the processing of this data. The values must be separated by commas.

#### Gross Vehicle Weight

GVWpdf.csv

This file holds the parameters of the distributions that characterize the GVW and speed of each class of truck for both directions. An example of this file is:

1,194.5,27.4,0.152,44.2,6.5,0.069,51.2,9.7,0.583,231.1,61.9,0.274,199.9,36.7

0,0,0,0.395,76.4,20.7,0.887,166.3,53.2,0.24,176.6,29.6,0.553,308.7,49.9

0,0,0,0.453,117.4,30.5,0.044,268.4,34.7,0.177,331,30.1,0.173,383.2,35.4

1,181.1,22.4,0.143,46.5,8,0.093,56.4,12.4,0.493,243.6,64.6,0.16,205.3,40.1

0,0,0,0.524,82.9,23.8,0.653,141.5,31.1,0.301,162.1,28.8,0.441,300.6,53.6

0,0,0,0.333,132.3,31.8,0.254,218.5,33.4,0.206,361.9,31.6,0.399,400.4,35.9

Again this is best explained by reference to the following table:

|  | Speed | 2-Axle GVW | 3-Axle GVW | 4-Axle GVW | 5-Axle GVW |
| --- | --- | --- | --- | --- | --- |
| Direction 1 | 3×3 | 3×3 | 3×3 | 3×3 | 3×3 |
| Blank Line |  |  |  |  |  |
| Direction 2 | 3×3 | 3×3 | 3×3 | 3×3 | 3×3 |

In the above table the entry 3×3 refers to the allowance for multi-modal distributions (up to a maximum of three modes) and includes, for each mode, the weight, mean and standard deviation, as explained previously. The values must be separated by commas.

#### Headway

NHM.csv

Of the headway models, only the HeDS model requires an input file. This model is defined in OBrien & Caprani (2005). An example is:

15,0,0,0

0,0.011855673,-0.014268241,0.004048786

0,0.039251526,-0.05978246,0.02212043

70,-0.004412997,0.054824101,-0.066907905

80,-0.004685721,0.052127816,-0.053475193

90,0.001537014,0.020896587,-0.013787689

100,-0.003853623,0.064555837,-0.069172155

110,-0.002530238,0.054511802,-0.059714977

120,-0.001307981,0.048010242,-0.051645258

130,-0.000487752,0.049738587,-0.057875119

140,-0.004995115,0.081041256,-0.086465967

150,-0.004547469,0.080310658,-0.083351351

160,-0.004938412,0.092219287,-0.105416601

170,-0.005000644,0.086893379,-0.097048852

180,0.001987438,0.052114614,-0.058245039

190,0.003366332,0.044909211,-0.063187142

210,0.000379907,0.068461437,-0.077769612

230,-0.006466786,0.117770005,-0.141174818

Line 1 indicates the number of flow-dependent headway models (always less than, or equal to, 24). Lines 2 and 3 give the parameters of the quadratic-fit headway cdf for under 1.0 s and between 1.0 s and 1.5 s respectively. The following lines (of number 15 in this example, from Line 1), return the parameters of the quadratic fit to the headway cdf for that flow (trucks per hour) of the first column. The values must be separated by commas.

### Constant Traffic Model (Model 1)

This is a simple test model in which the vehicles being generated are constant (i.e. no randomization). It is useful for verification of calculations and vehicle arrivals for example. Each car generated has 2axles of wheelbase 4.0 m, and weights 1 tonne each. And each truck is a 46 tonne 6-axle semi-trailer, with axle weights 7, 6, 6, and 3 × 9 tonnes, and spacings 3.5, 2.0, 6.0, 2 × 1.2 m. The proportion of cars to trucks is as per the traffic flow generation model.

### Vehicle Garage Model (Model 2)

This method of generating an artificial traffic stream, is a smoothed bootstrap using variable-bandwidth kernel density estimators approach (Enright, 2010, Scott, 2015). In this method, a vehicle is randomly selected from the garage of measured vehicles (bootstrapping), and then randomised (smoothing) using deviates (kernels). Thus, a very large number of artificial heavy vehicles in the traffic stream, including overloaded vehicles, can be generated, while maintaining intrinsic correlations and relationships within the vehicle database and characteristics (e.g. correlation between axle weights and gross vehicle mass).

The vehicle parameters modified using the kernels are the GVM, axle weights, and axle spacings. The literature presents two usual kernels: a gaussian and triangle kernel. Currently only a gaussian kernel is implemented with indicative parameters for the different vehicle characteristics as shown in the table below.

| Vehicle Characteristic | Bias | Standard Deviation (Bandwidth) |
| --- | --- | --- |
| Gross vehicle mass | 1 | 0.08 |
| Axle group mass | 1 | 0.05 |
| Axle group spacing | 1 | 0.02 |

#### Vehicle Garage File

This file is a vehicle file in one of the vehicle file formats (see Appendix). For good simulations it should contain sufficient numbers of each vehicle type to represent the entirety of traffic at the site. The proportions of each vehicle type should also reflect the relative proportions of vehicle types at the site.

#### Kernel File

The kernel file is a csv type formatted file, corresponding to the table above as follows:

1.0,0.08 1.0,0.05 1.0,0.02

## Configuration File

The user interacts with the program through the configuration file.

Important!

The input file must be called “BTLSin.txt” and it must be in the working folder.

An example input file is shown next, and each input line explained following.

| Line | BTLSin.txt |
| --- | --- |
| 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 | // --------------------------------------------- // START OF BRIDGE TRAFFIC LOAD SIMULATION INPUT // --------------------------------------------- // // --------------------------------------------- // *** INPUT SPECIFICATIONS *** // --------------------------------------------- // // Program Mode (1 - Gen & Sim, 2 - Gen, 3 - Read & Sim) 1 // // TRAFFIC GENERATION PARAMETERS // --------------------------------------------- // No. of days of traffic simulation: 20 // Location of Traffic folders C:\Users\ccapr\Google Drive\~Research\Workings\Traffic\Auxerre // Default truck track width (cm) 190.0 // Standard deviation of eccentricity in lane (cm) (about 20 cm usually) 0 // Vehicle Generation Model to be used // (0 - Grave, 1 - Test(Constant), 2 - Garage) 2 // Headway model to be used: // (0 - Auxerre NHM, 1 - Test(Constant), 5 - Congestion (w/ or w/out cars), 6 - free-flow, cars included) 6 // Traffic Classification System to be used: // (0 - No. of Axles, 1 - Axle Pattern) 1 // Lane and flow definition file: LaneFlowData_80.csv // Nominal congested spacing, front to back (m): 5 // Congested speed (km/h): 30 // Congested gaps coefficient of variation: 0.05 // // TRAFFIC GARAGE PARAMETERS // --------------------------------------------- // Garage file as basis for generation (in "Traffic input file format" below): garage.txt // Kernels (mean, std) file for randomizing the garage vehicles (lines in order GVW, AW, AS) kernels.csv // // TRAFFIC INPUT FILE PARAMETERS // --------------------------------------------- // Traffic input file to be analysed: WGBSlowLanes_Jul.txt // Traffic input file format (CASTOR - 1, BeDIT - 2, DITIS - 3, MON - 4): 4 // Impose constant speed on all vehicles (1 or 0): 0 // Use average speed of vehicles in file if constant speed imposed (1 or 0) 1 // Constant speed of vehicles if not average used (km/h): 80 // // LOAD EFFECT CALCULATION PARAMETERS // --------------------------------------------- // Bridge definition file: 1-ABT6111Bridges.txt // Influence Line definition file: 1-ABT6111ILS.txt // Influence Surface definition file: IS.txt // Time step (s): 0.1 // Minimum GVW for inclusion in calculations (t/10): 0 // // --------------------------------------------- // *** OUTPUT SPECIFICATIONS *** // --------------------------------------------- // // MISC. OUTPUT PARAMETERS // --------------------------------------------- // // Write full time history - slow & large file (1 or 0): 0 // Write each loading event value (1 or 0): 0 // Write each event buffer size: 10000 // Write a fatigue event file (1 or 0) 0 // // VEHICLE FILE // --------------------------------------------- // Write vehicle file (1 or 0) // WARNING: a large file may result in long-run simulations 0 // Traffic output file format (CASTOR - 1, BeDIT - 2, DITIS - 3, MON - 4): 4 // Vehicle file name NewVehicles.txt // Vehicle file buffer size 100000 // Write vehicle file flow statistics (1 or 0) 1 // // BLOCK MAXIMUM LOAD EFFECTS // --------------------------------------------- // Analyse for Block Max (overrides remaining params) (1 or 0) 1 // Block size for maxima (days): 1 // Block size for maxima (seconds): 0 // Write block max separated vehicle files (1 or 0): 1 // Write block max summary files (1 or 0): 1 // Do and write block max mixed vehicle analysis (1 or 0): 0 // Write block max buffer size: 2 // // PEAKS OVER THRESHOLD LOAD EFFECTS // --------------------------------------------- // Analyse for POT (overrides remaining params) (1 or 0) 0 // Write POT vehicle files (1 or 0): 0 // Write POT summary files (1 or 0): 1 // Write POT counter files (1 or 0): 1 // POT counter size (days): 1 // POT counter (seconds): 0 // Write POT buffer size: 10000 // // LOAD EFFECT STATISTICS OUTPUT // --------------------------------------------- // Analyse for Statistics (overrides remaining params) (1 or 0) 1 // Write cumulative statistics file (1 or 0) 1 // Write statistics at intervals files (1 or 0) 0 // Interval size for statistics output (seconds) 3600 // Write interval statistics buffer size: 10000 // // --------------------------------------------- // END OF BRIDGE TRAFFIC LOAD SIMULATION INPUT // --------------------------------------------- |

#### \\ Comments:

The program reads all lines of the configuration file except those preceded with C++ style commenting: “\\”. The user is free to add further commenting to the file as they wish, once the order of the input variables is not altered.

Important!

Depending on the program mode, some inputs are redundant. However, they must still be specified as ‘placeholders’ to keep the order of inputs the same.

#### Line 1:

The user specifies the program mode using 1, 2, or 3 for the modes as defined on page 15.

#### Line 2:

Specify the number of days of traffic to simulate in Modes 1 or 2. This input is redundant in the case of Mode 3 when the traffic file is specified.

#### Line 3:

Specify the location of the traffic generation files used in Program Modes 1 and 2 for the Grave Traffic Model (Model 0).

#### Line 4:

Specify the truck track width (axle width) to be used if there is no ATW.csv file for the site in the folder of Line 3 – only relevant for influence surface calculations.

#### Line 5:

Specify the standard deviation of the eccentricity of the vehicles in their lane. This approach assumes a mean of zero and a normal distribution. Some site data suggests about 20 cm is a reasonable value. Again, only applies to influence surface calculations.

#### Line 6:

Vehicle generation model to be used:

- Model 0: The Grave Model, as described in Caprani (2005);

- Model 1: A constant vehicle model, one car and one truck type;

- Model 2: Bootstrap from vehicle garage using kernels.

#### Line 7:

Specify the headway model to be used in the generation of artificial traffic. The options are (note the odd-numbering for ‘historical’ reasons):

- “0” – The HeDS (Headway Distribution Statistics) model of OBrien & Caprani (2005). This is suitable for the Auxerre site-measured flowrates only. It is a free-flow model that generates only trucks.

- “5” – Congestion model as per Caprani (2012), summarized in the following diagram. A nominal axle gap is specified (Line 6), along with a coefficient of variation between successive vehicles in all lanes (i.e. trucks and cars) (Line 8) and gaps are then generated using a normal distribution.

- “6” – Free-flow model which uses a Poisson arrival assumption based upon the Normalized Headway Model of Crespo-Minguillón and Casas (1997). This accounts for different flow rates (Q) as follows:

Where  is the mean headway, i.e. the average time gap between vehicles in the hour of the current total (cars and trucks) flowrate Q and so is given by Q/3600.

For all models the program checks that no overlapping of vehicles can occur by ensuring the generated gap is greater than the required minimum gap (taking account of the maximum bridge length, vehicle lengths, and speed difference between them).

#### Line 8:

The traffic classification scheme to be used in output:

- 0 – classify by the number of axles

- 1 – classify by the axle pattern, where axle groups are identified as consecutive axles with spacing < 2.1 m.

#### Line 9:

Specify the name of the Lane Flow definition file that is in the working folder.

#### Line 10:

Specify the nominal axle gap for Headway Model 5 – congestion.

#### Line 11:

Specify the speed of all vehicles for Headway Model 5 – congestion. Note that this, combined with the calculation time step (Line 16), effectively renders a distance-stepping algorithm and so this speed can be notional to achieve a required distance step.

#### Line 12:

Specify the coefficient of variation of the nominal congested gap for Headway Model 5 – congestion.

#### Line 13:

Specify the filename in the working folder for the garage of vehicles to be used for Traffic Generation Model 2. Note, this file must be in the format of the Traffic Input File specified in Line 16.

#### Line 14:

Specify the filename in the working folder of the kernel specifications for the Traffic Generation Model 2.

#### Line 15:

Specify the name of the traffic input file in the working folder for Program Mode 3.

#### Line 16:

Specify the format of the input file: 1 – CASTOR format, 2 – BeDIT, 3 – DITIS format, 4 – MON. See Appendix for traffic file format definitions.

#### Line 17:

BTLS normally passes each vehicle across the bridge according to its own speed when in Program Mode 3. Setting this option to “1” imposes constant speed on all vehicles for comparison with some other algorithms – mostly to do with congestion traffic files.

#### Line 18:

When constant speed is imposed (Line 11), if this option is “1” then the average speed of all vehicles in the file will be used, otherwise the speed specified in Line 19 will be used.

#### Line 19:

Specifies the constant speed if Line 17 is “1” and Line 18 is “0”.

#### Line 20:

Specify the name of the bridge definition file (see Section 0) in the working folder.

#### Line 21:

Specify the name of the influence line definition file (see Section 3.6) in the working folder.

#### Line 22:

Specify the name of the influence surface definition file (see Section 3.7) in the working folder.

#### Line 23:

Specify the calculation time step which is used in passing the vehicles over the bridges. 0.1 s has been found a good compromise between accuracy and efficiency. For some very sharp influence lines (e.g. shear forces) a finer step may be required. A sensitivity study is recommended.

#### Line 24:

To avoid unnecessary computation of smaller vehicles, this specifies the minim GVW for a vehicle’s load effect to be calculated. Its spatial arrangement on the road is not affected if its GVW is less than this number. The units are deci-tonnes (t/10)

#### Line 25:

Specify “1” to write a full time history of the load effects – see section on BTLS Output for more details. This should be set to “0” for long simulations due to enormous resulting file size and slow execution.

#### Line 26:

Specify “1” to write the load effect value for each loading event that occurs. Again this can be a large file and cause slow execution for long-run simulations. See section on BTLS Output for more details.

#### Line 27:

If each loading event value is to be written (Line 24), this option if set to “1” specifies the number of events that are stored in memory before writing to the hard drive. See section on BTLS Output for more details (Section 5).

#### Line 28:

Writes a file suitable for further fatigue calculations, giving load cycles.

#### Line 29:

For all program modes, this option if “1” will write the generated or read-in vehicle file. For long run simulations this should be “0” as very large files can result, filling hard drive space and causing very slow computation. Mostly useful for short debugging or test runs.

#### Line 30:

This specifies the format of the output file traffic file: 1 – CASTOR format, 2 – BeDIT, 3 – DITIS, 4 – MON.

#### Line 31:

The name of the file to be written if Line 29 is “1”.

#### Line 32:

If the vehicle file is to be written (Line 29 set to “1”), this specifies the number of vehicles that are stored in memory before writing to the hard drive. See section on BTLS Output for more details.

#### Line 33:

If this is set to “1” files are output giving the traffic flow and composition information for each hour of the simulation for each lane.

#### Line 34:

If this option is “1” calculations are performed that can be used to write block maxima output. Set to “0” to override all block maxima output and calculations.

#### Line 35:

For block maximum output, this specifies the block size in days for which the maximum is retained (it can be zero if Line 36 has a number > 0).

#### Line 36:

For block maximum output, this specifies the block size in seconds for which the maximum is retained (it can be zero if Line 35 has a number > 0).

#### Line 37:

Specify “1” to write block maximum load effect and vehicle output files for each number of vehicles comprising the events. See section on BTLS Output (Section 5.4) for more details.

#### Line 38:

Specify whether to write the block maximum summary files (“1” or “0”). See section on BTLS Output (Section 5.4) for more details.

#### Line 39:

Specify “1” to write block maximum load effect output files for which the events are not separated by the number of vehicles in the event, or “0” to not. See section on BTLS Output (Section 5.4) for more details.

#### Line 40:

If block maximum output is to be written (Line 34), this option if set to “1” specifies the number of events that are stored in memory before writing to the hard drive. See section on BTLS Output (Section 5.4) for more details.

#### Line 41:

If this option is “1” calculations are performed that can be used to write peaks-over-threshold (POT) output. Set to “0” to override all POT output and calculations. Note that the thresholds for each load effect are set in the Bridge Definition File (Section 0).

#### Line 42:

For POT output, this specifies if the vehicles comprising the peak events are to be output to a vehicle-event file. See section on BTLS Output (Section 5.5) for more details.

#### Line 43:

For POT output, this specifies if summary files are to be written. See section on BTLS Output (Section 5.5) for more details.

#### Line 44:

For POT output, this specifies if a peaks counter file is to be written. See section on BTLS Output (Section 5.5) for more details.

#### Line 45:

For peaks counter output, this specifies the interval size in days for which the peaks are counted (it can be zero if Line 46 has a number > 0).

#### Line 46:

For peaks counter output, this specifies the interval size in seconds for which the peaks are counted (it can be zero if Line 45 has a number > 0).

#### Line 47:

If POT output is to be written (Line 41), this option if set to “1” specifies the number of events that are stored in memory before writing to the hard drive. See section on BTLS Output (Section 5.5) for more details.

#### Line 48:

If this option is “1” calculations are performed that accumulate simple statistics of load effect and vehicles throughout the simulation. Set to “0” to override all statistics output and calculations.

#### Line 49:

This specifies if the statistics for each load effect accumulated through the whole simulation are to be output. See section on BTLS Output (Section 5.6) for more details.

#### Line 50:

This specifies if the statistics for each load effect are to be output at particular time intervals. See section on BTLS Output (Section 5.6) for more details.

#### Line 51:

If interval statistics are to be output, this specifies the interval duration. See section on BTLS Output (Section 5.6) for more details.

#### Line 52:

If interval statistics are to be output, this specifies the number of intervals that are stored in memory before writing to file.

## Bridge Definition File

The bridges over which vehicles are to pass are defined in the bridge definition file, specified in the configuration file (BTLSin.txt). The file name is arbitrary.

Important!

A bridge definition file must be included in the working folder.

An example bridge definition file for a single bridge with 4 load effects is shown:

| Line No. | File |
| --- | --- |
| 1 2 3 4 5 6 7 8 9 10 | 1, 40.0, 2, 4 1, 1, 3000.0 1, 1, 0.728125, 0.271875 2, 1, 3000.0 2, 1, 0.728125, 0.271875 3, 2, 3000.0 2, 2, 0.728125 2, 2, 0.271875 4, 3, 3000.0 1 |

Any number of bridges can be defined, as can any number of load effects for each bridge. Each bridge definition is formatted as follows:

#### Bridge Information: (e.g. “1,	40.0, 2, 4”)

This first line specifies some general information about the bridge:

- Column 1: the bridge number, a positive integer – 1 in this case;

- Column 2: the span of the bridge in metres, a real positive number – 40.0 m in this case;

- Column 3: the number of lanes on the bridge, a positive integer – 2 in this case;

- Column 4: the number of load effects to be considered for this bridge, a positive integer – 4 in this case.

Each load effect is then defined on the following lines with a format depending on the type of load effect. The first line in each case is the basic information, formatted as follows:

#### Load Effect Information: (e.g. “1, 	1, 	3000.0”)

- Field 1: the load effect number, a positive integer – e.g. 1.

- Field 2: the load effect definition type:

  - 1 – lane factors are applied to a single influence line;

  - 2 – separate influence lines and lane weights are applied to each lane;

  - 3 – an influence surface is used for the load effect.

- Field 3: the threshold to be used for this load effect in peaks-over-threshold analysis, e.g. 3000.0 kNm. If this number is absent it is assumed to be zero.

The line(s) following the load effect information line are formatted differently depending on the load effect definition type, 1, 2, or 3, as follows:

Load Effect Definition Type 1 – Lane factors applied to single IL

Only one line is required in this case, for example “1, 1, 0.728125, 0.271875” (line 3) in the file above. This line is formatted as follows:

- Field 1: The type of influence line:

  - 1 – if it is a built-in influence line;

  - 2 – if it is a user-defined discrete influence line.

- Field 2: The influence line number (whether built-in or discrete).

- Fields3+: the lane factors to be applied to the influence line for Lane 1, 2 etc.

Line 5 in the file above shows that a discrete influence line is being used for Load Effect 2 with lane factors for all lanes.

Load Effect Definition Type 2 – Separate ILs and lane weights for each lane

In this case a line is required, corresponding to each lane, formatted as follows:

- Field 1: The type of influence line:

  - 1 – if it is a built-in influence line;

  - 2 – if it is a user-defined discrete influence line.

- Field 2: The influence line number (whether built-in or discrete).

- Fields3: the lane factor to be applied to the influence line for this lane.

In the example file above, load effect 3, starting on line 6, is using this form of definition. Line 7 defines that Lane 1 uses discrete IL number 2 with lane weight 0.728125. Similarly, line 8 defines that Lane 2 uses the same discrete IL number 2 with lane weight 0.271875.

Load Effect Definition Type 3 – An influence surface is used for the bridge

In this case only a single line is required, assigning an influence surface number to the load effect. Thus, in the above file, line 9 shows that load effect 4 is using an influence surface, whilst line 10 assigns influence surface 1 to it.

Lane factors represent the proportion of load of the corresponding lane (i.e. lane factor 3 is for lane 3) that contributes to the load effect in the element under consideration. In this way, an influence surface is effectively defined as slices along each lane. Note that this model means that the influence surface must be a scaled version of itself transversely across the bridge – this is not always the case however.

Finally, it can be noted that the above example file is a test file: all of the load effects use different means of arriving at the same outcome once the discrete influence line is for mid-span bending moment, and the influence surface is that given as the example influence surface later on.

### Built-In Influence Functions

The built-in influence functions are mathematical expressions that apply for any bridge length and can be weighted with any value of lane factor. Consequently, these built-in functions execute more quickly than read-in influence lines.

The description and index for the built in functions are:

| Index | Influence Line | Location |
| --- | --- | --- |
| 1 | Mid-span bending moment for a simply supported beam | B |
| 2 | Bending moment over the central support of a two-span beam | E |
| 3 | Left-hand shear in a simply-supported beam | A |
| 4 | Right-hand shear in a simply-supported beam | C |
| 5 | Right-hand shear for a two-span beam | F |
| 6 | Left-hand shear for a two-span beam | D |
| 7 | Total amount of load on the bridge (i.e. the unit influence line) |  |

A

B

C

D

E

F

## Lane Flow Data File

This file contains all information relating to the number of lanes, the flow in each lane throughout the day, and the traffic composition. It applies when artificial traffic is being created using one of the free-flow headway models.

Important!

A lane flow definition file must be included in the working folder.

This file must be in *.csv format (but can have a *.txt extension). In *.csv format it is easily edited in a spread sheet program as shown:

Each lane of the simulation is defined by 25 rows of data:

- The first row defines the lane number (sequential) and direction number (1 or 2) in columns 1 and 2 (or A and B in the screenshot);

- The next 24 rows describe the traffic flow for each hour of the day (i.e. rows 2, 3, 4… in the screenshot above).

Note that it is assumed that every day of the simulation is the same. Typically only economic days of traffic are simulated of 5 days per week, 50 weeks per year (250 days per year). It is assumed that each such day has the same properties.

The structure of each hour description is as follows, with numbers given from hour 0 of the screen shot above (at row 2):

- Column 1: The hour identifier, starting at midnight, 0 to 23 (e.g. 0)

- Column 2: The mean truck flow rate in this hour (trucks/hour) (e.g. 153.8)

- Column 3: Mean velocity of traffic (dm/s) (e.g. 248)

- Column 4: Standard deviation of the velocity (dm/s) (e.g. 10)

- Column 5: Percentage of cars in this traffic model (e.g. 80)

- Column 6: Percentage of trucks that are 2 axle (e.g. 23)

- Column 7: Percentage of trucks that are 3 axles (e.g. 2.8)

- Column 8: Percentage of trucks that are 4 axles (e.g. 31.7)

- Column 9: Percentage of trucks that are 5 axles (e.g. 42.5)

The rationale for having the input in this form is the ease of altering the percentage cars and overall flow rate without modifying the truck flow and composition. This is best explained through an example of the calculations the program performs:

- From Column 5, the truck percentage is 100-80 = 20%;

- This 20% represents a flow of 153.8 vehicles per hour (Column2);

- Thus the total flow rate is 153.8/0.2 = 769 vehicles per hour.

- Of the 20% vehicles that are trucks, for example, 42.5% are 5-axle trucks, thus there will be 0.425*153.8 = 65.4 5-axle trucks on average for this hour.

Changing Column 4 then changes the overall flow rate, without changing the number of trucks that arrive.

Note that a normal distribution is assumed for the speed of all vehicles.

Each lane to be included in the simulation must have the above information. An example file with two lanes, one in each direction is given below:

1,1,,,,,,,

0,153.8,248,10,80,23,2.8,31.7,42.5

1,131,248,10,80,23,2.8,31.7,42.5

2,131.8,248,10,80,23,2.8,31.7,42.5

3,123.8,248,10,80,23,2.8,31.7,42.5

4,114,248,10,80,23,2.8,31.7,42.5

5,121.2,248,10,80,23,2.8,31.7,42.5

6,141.2,248,10,80,23,2.8,31.7,42.5

7,155.4,248,10,80,23,2.8,31.7,42.5

8,154,248,10,80,23,2.8,31.7,42.5

9,141,248,10,80,23,2.8,31.7,42.5

10,126.4,248,10,80,23,2.8,31.7,42.5

11,101.6,248,10,80,23,2.8,31.7,42.5

12,95.8,248,10,80,23,2.8,31.7,42.5

13,88.2,248,10,80,23,2.8,31.7,42.5

14,93,248,10,80,23,2.8,31.7,42.5

15,109,248,10,80,23,2.8,31.7,42.5

16,124.2,248,10,80,23,2.8,31.7,42.5

17,151,248,10,80,23,2.8,31.7,42.5

18,141.8,248,10,80,23,2.8,31.7,42.5

19,172.2,248,10,80,23,2.8,31.7,42.5

20,141.4,248,10,80,23,2.8,31.7,42.5

21,148,248,10,80,23,2.8,31.7,42.5

22,157.2,248,10,80,23,2.8,31.7,42.5

23,159.4,248,10,80,23,2.8,31.7,42.5

2,2,,,,,,,

0,92.2,222,10,80,21.9,2.3,31,44.8

1,79.6,222,10,80,21.9,2.3,31,44.8

2,67,222,10,80,21.9,2.3,31,44.8

3,74.8,222,10,80,21.9,2.3,31,44.8

4,81.6,222,10,80,21.9,2.3,31,44.8

5,94.8,222,10,80,21.9,2.3,31,44.8

6,102.4,222,10,80,21.9,2.3,31,44.8

7,121.2,222,10,80,21.9,2.3,31,44.8

8,127.4,222,10,80,21.9,2.3,31,44.8

9,127.2,222,10,80,21.9,2.3,31,44.8

10,112.2,222,10,80,21.9,2.3,31,44.8

11,111.4,222,10,80,21.9,2.3,31,44.8

12,110.2,222,10,80,21.9,2.3,31,44.8

13,146,222,10,80,21.9,2.3,31,44.8

14,160.4,222,10,80,21.9,2.3,31,44.8

15,152,222,10,80,21.9,2.3,31,44.8

16,151,222,10,80,21.9,2.3,31,44.8

17,167.2,222,10,80,21.9,2.3,31,44.8

18,179.6,222,10,80,21.9,2.3,31,44.8

19,164.8,222,10,80,21.9,2.3,31,44.8

20,206.6,222,10,80,21.9,2.3,31,44.8

21,228.4,222,10,80,21.9,2.3,31,44.8

22,189,222,10,80,21.9,2.3,31,44.8

23,138.8,222,10,80,21.9,2.3,31,44.8

## Influence Line Definition File

This file stores the definitions of any discrete influence lines that are required. It must be in *.csv format (but can have a *.txt extension).

Important!

An influence line definition file must be included in the working folder if it is required for the analysis.

The first line of the file is the number of influence lines defined within. Subsequently, each influence line is defined with the following structure:

- A first line giving the influence line number (Column 1) and the number of points defining the influence line (Column 2);

- Subsequent lines define the influence line using x, y, pairs for the location and ordinate values (Columns 1 and 2 respectively).

Discrete influence line processing takes longer than built-in expressions. The program must search the vector of x-coordinates to find the points surrounding the axle location. Linear interpolation of the ordinates is then used to find the ordinate at the axle location. The spacing of points need not be uniform. Therefore, prefer to use as few points as is necessary where the influence line is linear, and more points where it is curved.

Important!

The program warns if the last x-coordinate is not the same as the length of the bridge defined in the Bridge Definition file. Behaviour in this case is generally unpredictable. However this warning can be issued due solely to rounding, and in this case no problems have been observed.

An example file is given below:

- he first influence line is a test of a 40 m simply-supported mid-span bending moment calculation,

- the second is an influence line from the Millau viaduct, courtesy if IFSTTAR, France.

2

1, 13

0.000000,    0.000000

3.333333,    4.695709

6.666667,    9.391137

10.000000,   14.086847

13.333333,   18.782556

16.666667,   23.477984

20.000000,   28.173693

23.333333,   23.477984

26.666667,   18.782556

30.000000,   14.086847

33.333333,    9.391137

36.666667,    4.695709

40.000000,    0.000000

2, 67

0.000000,    0.000000

3.200000,   -1.404704

6.400000,   -2.482683

9.600000,   -2.967398

12.800000,   -2.765344

16.000000,   -1.658346

19.200000,    0.578218

22.400000,    4.170048

25.600000,    9.341767

28.800000,   16.319074

32.000000,   25.326593

35.200000,   36.574975

38.400000,   50.127631

41.600000,   31.722458

44.800000,   15.056238

48.000000,    0.000000

51.333333,  -13.793401

54.666667,  -25.918783

58.000000,  -36.084887

61.333333,  -44.196060

64.666667,  -49.871840

68.000000,  -52.900498

71.333333,  -53.815114

74.666667,  -52.833863

78.000000,  -50.155575

81.333333,  -46.221973

84.666667,  -41.193196

88.000000,  -35.436810

91.333333,  -29.230102

94.666667,  -22.847134

98.000000,  -16.583464

101.333333,  -10.646519

104.666667,   -5.083589

108.000000,    0.000000

111.333333,    4.397896

114.666667,    8.216499

118.000000,   11.460109

121.333333,   14.100781

124.666667,   15.951508

128.000000,   16.941357

131.333333,   17.243363

134.666667,   16.930610

138.000000,   16.067582

141.333333,   14.799371

144.666667,   13.179716

148.000000,   11.327914

151.333333,    9.335319

154.666667,    7.293285

158.000000,    5.293167

161.333333,    3.385477

164.666667,    1.617506

168.000000,    0.000000

171.200000,   -1.360639

174.400000,   -2.545019

177.600000,   -3.539167

180.800000,   -4.268925

184.000000,   -4.746116

187.200000,   -4.978264

190.400000,   -4.989011

193.600000,   -4.803078

196.800000,   -4.444110

200.000000,   -3.936826

203.200000,   -3.307020

206.400000,   -2.577262

209.600000,   -1.773345

212.800000,   -0.904943

216.000000,    0.000000

## Influence Surface Definition File

This file stores the definitions of any influence surfaces that are required. It must be in *.csv format (but can have a *.txt extension).

Important!

An influence surface definition file must be included in the working folder if it is required for the analysis.

Any number of influence surfaces can be defined in the file. Each influence surface is defined on a rectangular grid according to a specific format as follows:

- The first line defines the basic data as follows:

  - Field 1: the influence surface number;

  - Field 2: the number of rows of data (the number of x- or longitudinal coordinates) specifying the influence surface grid points;

  - Field 3: the number of lanes on the bridge/influence line

  - Field 3+i: the location of the edge of lane i on the influence surface;

  - Last Field: the upper edge of the last lane.

- Line 2 defines the y- or transverse coordinates of the grid points.

- Each row then defines the ordinates at the grid points, the first value of which is the x-coordinate.

It is taken that Direction 1 traffic travels in the positive x-direction. For load application between grid points, linear interpolation is used in both directions. Hence, for flat areas of the influence surface a larger grid size can be used, but for more peaked zones, a more refined grid is required.

An example file is shown below in which a single test influence surface for mid-span bending moment of an edge beam is defined for comparison with a standard method using built-in or discrete influence lines and lane factors. The following figure describes the file and its definitions.

1,5,2,0.35,4.0,7.65

0,0,2,4,6,8

0,0,0,0,0,0

10,5,3.75,2.5,1.25,0

20,10,7.5,5,2.5,0

30,5,3.75,2.5,1.25,0

40,0,0,0,0,0

Important!

If multiple influence surfaces are defined for a bridge, the program assumes that the lane coordinates are the same for all influence surfaces. This is as it should be for a single real bridge of course. However, different lane coordinates can be used with different influence surface definitions for different bridges under the same traffic, either read or generated.

# Using BTLS

Running the program

The program can be run from any folder as explained previously. An example working folder showing all files necessary to execute the program is given below:

Note that the 64 bit version is being used here.

Important!

To run the program, double click the executable (*.exe) file.

## Console output

Some examples of console output during program execution are given.

### Example 1: Program Mode 1

After each day of simulation is complete, the program outputs a notice. At the end of the simulation the elapsed time is displayed for information. In this case, 20 days of traffic and generated and simulated crossing 5 bridges, each with 3 load effects.

### Example 1: Program Mode 2

Ten days of vehicles are generated and the current day number is given (right hand columns). Each time the program flushed the vehicle buffer to the file, an output is given of the simulation time at which it occurred.

The fodler is then populated with the outputted vehicle file as named in “BTLSin.txt”.

### Example 3: Program Mode 3

The traffic file created in the last example is read in and passed over 5 bridges, each with 3 load effects. The output is as follows:

Note the slower execution time than for Program Mode 1 which has 20 days of traffic. This is caused by the additional overhead required to manipulate the 25 MB traffic file that has been read into memory.

## Input Errors

When there are errors in the input, the behaviour is unpredictable. More informative user feedback will be built in soon.

Some potential problems:

- The supporting files (e.g. bridge, lane flow, IL) cannot be found in the working folder;

- The traffic folder cannot be found (it should be at “C:\Traffic”);

- The vehicle file to be read in cannot be found (Program Mode 3).

- Outputs are not matched to Program Mode (e.g. Program Mode 2 – Generate vehicle file, but no vehicle file is to be output – Line 20 of BTLSin.txt.)

In each of these cases, the program may:

- Output helpful warnings;

- Flash open and close immediately;

- Remain open and display unusual text, such as “Conversion Error”.

Admittedly, all behaviours should be of the first type – this will improve.

# BTLS Output

Introduction

BTLS can produce large amounts of output, especially for long run simulations or heavily congested traffic. Accessing the hard drive often can significantly slow the program’s execution. Therefore, keep buffer sizes as large as memory and execution speed can allow.

All outputs are in text file format with specific information layouts and formats for each type of output file.

### Effect on Execution Speed

Obviously, the more outputs that are needed the slower the simulation. Some general comments to aid execution time are:

- Only output what is needed: this can be ascertained by a few short runs before doing the main long-run simulation.

- Use the buffer size variables to good effect: for sample runs monitor the program’s RAM usage, increasing the buffer sizes as much as possible to prevent undue writing to disc, one of the slowest operations.

- The statistics output is intended mainly for short runs to give information for peaks-over-threshold analysis (i.e. threshold levels). Consequently, turn it off when it is not required.

- Similarly, the flow data output is only intended for short-run verification that the traffic is being properly modelled. Consequently, turn it off when it is not required.

- For the Block Max and POT outputs, turn off the vehicle output files if they are not needed.

## Miscellaneous Output

### Time History File

If a full time history is selected to be output (Line 25 of BTLSin.txt), BTLS creates a single file for each bridge named:

TH_L.txt

Where L is the bridge length. The file gives the load effect at each time step of the simulation for each load effect considered for the bridge. Note that no output is given when there is zero load effect. A sample output is:

The format is:

- Column 1: the current time. Note that time starts at the time of arrival of the first vehicle;

- Column 2: The number of trucks currently on the bridge;

- Columns 3+: The current value of each load effect is given, according to the order of the load effects in the bridge definition file.

This file can get extremely large, but for short runs (e.g. 1-day) it is very useful for checking and debugging output.

An example output is given showing the number of trucks on the bridge through the day. As can be seen, four 3-truck events occur on this 20 m bridge.

### All Events File

If all loading events are selected to be output (Line 26 of BTLSin.txt), BTLS creates a single file for each bridge named:

BL_L_AllEvents.txt

Where L is the bridge length. The file gives the maximum of each calculated load effect recorded during each loading event to occur. A loading event is defined as the loading that occurs between two occasions of zero trucks on the bridge, or the departure or arrival of another vehicle. A sample output is:

The format is:

- Column 1: the starting time of the loading event.

- Column 2: The number of trucks in the event;

- Columns 3+: The maximum value of each load effect during the event.

This file can get extremely large, but is useful for checking and debugging output.

### Fatigue Events File

If fatigue events are selected to be output (Line 28 of BTLSin.txt), BTLS creates a single file for each bridge named:

BL_L _Fatigue.txt

Where L is the bridge length. The file gives the maximum and minimum values of each loading event (defined above) in chronological order. This is suitable for examination of fatigue cycles. A sample output is shown below:

The format is:

- Column 1 (Line 1): the starting time of the loading event;

- Column 2 (Line 2): The number of trucks in the event.

For each line, the subsequent columns are:

- Column 3: The time at which the value of load effect 1 is recorded;

- Column 4: the value of load effect 1.

Columns 5 & 6 give the time and value of load effect 2, and so on.

For example, this file shows that load effect 3 had a maximum value at 17.04 s, followed by a minimum at 17.74 s.

## Vehicle Output

### Traffic File

A traffic file can be output in any Program Mode if selected on Line 29 of BTLSin.txt. For Program Mode 2, this must be selected.

The file is named as specified on Line 31 of BTLSin.txt.

The program outputs vehicles in CASTOR format. An example of the program output for several trucks is given:

1001 1 1 2 0 12618155  54 43211 18 2743 27 0  0 0  0 0  0 0  0 0  0 0  0 0  0

1001 1 1 2 0 2 412133 137 67311 18 5441 6626 17 0  0 0  0 0  0 0  0 0  0 0  0

1001 1 1 2 0 2 598157  64 43211 18 3243 32 0  0 0  0 0  0 0  0 0  0 0  0 0  0

1001 1 1 2 0 44354134 336133511 18 7839 7040 6327 6327 63 0  0 0  0 0  0 0  0

1001 1 1 2 0 93062152 117 67311 18 3941 3926 39 0  0 0  0 0  0 0  0 0  0 0  0

1001 1 1 2 0101765131  97 44211 18 5344 44 0  0 0  0 0  0 0  0 0  0 0  0 0  0

### Flow Statistics

For generated or read-in vehicle files, selecting this option will output files containing the flow rate and traffic stream composition. The files are named:

FlowData_D_L.txt

Where D is the direction number and L is the lane number corresponding to those in the Lane Flow Data definition file.

For each hour of the simulation, the following statistics are collected for each lane:

- The number of vehicles in the hour

- The number of trucks;

- The number of cars;

- The number of 2-axle, 3-axle, 4-axle and 5-axle trucks in the hour.

This output allows direct comparison with the input given in the Lane Flow Definition file is generating traffic. A screenshot is shown below:

## Block Maximum Files

If block maximum vehicle files are selected to be output (Line 34 of BTLSin.txt), BTLS can creates different types of file output.

### Output by Number of Trucks

This option is specified on Line 37 of BTLSin.txt. Output is given for each load effect of each bridge named:

BM_V_L_N.txt

Where L is the bridge length and N is the number of trucks comprising the loading events. In this manner a comprehensive breakdown of the causing of loading can be studied, suitable for application of the CDS method (Caprani et al 2008).

A sample output is shown:

This file structure is termed a Loading Event File, and its structure explained later.

### Summary Files

If block maximum summary files are selected to be output (Line 38 of BTLSin.txt), BTLS creates a file for each load effect of each bridge named:

BM_S_L_Eff_E.txt

Where L is the bridge length and E is the load effect number. The file gives the maximum load effect recorded during each block, broken down according to the number of trucks comprising the event. A sample output is:

The format is:

- Column 1: The block index.

- Column 2: The block maximum 1-truck load effect;

- Columns 3: The block maximum 2-truck load effect

- Columns 4+: As appropriate, the block maximum 3-, 4-, … truck load effect.

Taking the maximum across Columns 2+ gives the overall block maximum load effect.

### Mixed Vehicle Output

This option is specified on Line 39 of BTLSin.txt. This form of output represents the conventional form in which the number of trucks comprising the event is not taken into account. Instead, the maximum load effect recorded during the block is noted. One such file per bridge is output, named:

BM_V_L_All.txt

Where L is the bridge length. This file structure is a Loading Event File, explained later. A sample output showing 3 blocks is:

As can be seen, there are a different number of trucks comprising the loading events that cause the maximum of each load effect in the block. Sometimes the same truck(s) loading event causes the maximum of 2 or more load effects, but in general different loading events cause the maximum of each load effect. This is because of the different shapes of the influence lines, and hence critical loading arrangements.

## Peaks-Over-Threshold Files

If POT files are selected to be output (Line 41 of BTLSin.txt), BTLS can creates two types of file output.

### Vehicle Files

This option is specified on Line 42 of BTLSin.txt. The vehicles comprising the loading event that is a peak are output in a Loading Event File structure. The files are named:

PT_V_L_E.txt

Where L is the bridge length and E is the load effect number. This file structure is a Loading Event File, explained later. A sample output showing 2 peaks is:

Note that the number of vehicles comprising the loading events are mixed and not separated out.

### Summary Files

This option is specified on Line 43 of BTLSin.txt. The vehicles comprising the loading event that is a peak are output in a Loading Event File structure. The files are named:

PT_S_L_Eff_E.txt

Where L is the bridge length and E is the load effect number. Each row of data in this file corresponds to a recorded peak. For each peak, the following data is ouput:

- The peak number;

- The time at which the peak occurred;

- The number of truck sin the event;

- The peak load effect value.

A screenshot of such a file is shown below.

Note that since there is not much memory required to store peak information, the buffer size defined on Line 36 of BTLSin.txt should be large to prevent frequent disk writing which is slow.

### Counter Files

This option is specified on Line 44 of BTLSin.txt. The number of peaks over the specified threshold occurring in each counter interval (defined on Lines 45-6 of BTLSin.txt) is output for each load effect. The files are named:

PT_C_L.txt

Where L is the bridge length. Each row of data in this file corresponds to a counter interval, and for each block and load effect the number of peaks is given. A screenshot of such a file is shown below.

If the POT output buffer size (Line 47 of BTLSin.txt) is such that an output occurs during a counter interval, then the interval will have two rows (the number of peaks before output, and number after output) and the total number of peaks will be the sum of the two results. Again, since there is not much memory required to store peak information, the buffer size defined on should be large to prevent frequent disk writing which is slow.

## Load Effect Statistics Output

BTLS can output some useful summary statistics of the calculated load effects. The statistics currently supported are:

- Events count;

- The number of vehicles recorded;

- The number of trucks recorded

- The minimum load effect value;

- The maximum load effect value;

- The mean load effect value;

- The standard deviation of load effect;

- The load effect variance;

- The load effect skewness;

- The load effect kurtosis;

- The number of events comprising 1, 2, ands on, trucks.

### Cumulative Statistics

For this output, the statistics are accumulated throughout the full length of the simulation. The files are named:

SS_C_L.txt

Where L is the bridge length. In this file, each row corresponds to a load effect. A sample screenshot is:

### Interval Statistics

For intervals of specified duration (Line 51 of BTLSin.txt), the statistics are recorded and written to disk when the buffer size is exceeded (Line 41 of BTLSin.txt). This buffer size should be quite large since not much memory is needed to store statistics. The files are named:

SS_S_L_Eff_E.txt

Where L is the bridge length and E is the load effect number. A sample output is shown below. The interval number and time are also given.

## Loading Event File Structure

This file structure contains all information relating to the event. An example is shown below (from the Block Maximum Vehicle Files, Out by Number of Trucks screenshot).

| Line | Data |
| --- | --- |
| 1 | 1 |
| 2 | 1 1475.9 84395.5 17.58 2 |
| 3 | 1001 1 1 023263466221 521119522 18 833315562 9413 9412 94 0 0 0 0 0 0 0 0 |
| 4 | 1001 1 1 023263478251 593113511 18 8331167551151311513115 0 0 0 0 0 0 0 0 |
| 5 | 2 409.0 84395.4 15.07 2 |
| 6 | 1001 1 1 023263466221 521119522 18 833315562 9413 9412 94 0 0 0 0 0 0 0 0 |
| 7 | 1001 1 1 023263478251 593113511 18 8331167551151311513115 0 0 0 0 0 0 0 0 |
| 8 | 3 378.4 84395.3 12.56 2 |
| 9 | 1001 1 1 023263466221 521119522 18 833315562 9413 9412 94 0 0 0 0 0 0 0 0 |
| 10 | 1001 1 1 023263478251 593113511 18 8331167551151311513115 0 0 0 0 0 0 0 0 |
| 11 | 2 |
| 12 | 1 1198.8 100764.2 26.65 2 |
| 13 | 1001 2 1 0 3592311240 449115511 18 683311857 8812 8813 88 0 0 0 0 0 0 0 0 |
| 14 | 1001 2 1 0 3592372235 609108511 18 9429187561091010912109 0 0 0 0 0 0 0 0 |
| 15 | 2 324.5 123199.1 -1.71 2 |
| 16 | 1001 2 1 0101318 9222 219 61222 18 7461145 0 0 0 0 0 0 0 0 0 0 0 0 0 0 |
| 17 | 1001 2 1 010131847242 600110511 1811030140581171211710117 0 0 0 0 0 0 0 0 |
| 18 | 3 426.2 100764.2 26.65 2 |
| 19 | 1001 2 1 0 3592311240 449115511 18 683311857 8812 8813 88 0 0 0 0 0 0 0 0 |
| 20 | 1001 2 1 0 3592372235 609108511 18 9429187561091010912109 0 0 0 0 0 0 0 0 |

Each line is explained as follows.

#### Line 1:

The index of the current block (if block maximum output), or the index of the particular loading event in legacy files.

#### Line 2:

This is the load effect information line with 5 fields of data separated by tabs:

- Field 1: The load effect number;

- Field 2: The value of the load effect;

- Field 3: The time at which this load effect was found in seconds;

- Field 4: The distance of the first axle of the first truck on the bridge relative to the bridge datum, at the time of the crossing event maximum effect being reached. This allows one to sketch the positions of the trucks at the time of the load effect.

- Field 5: The number of trucks comprising the event.

#### Line 3-4:

These lines provide the truck data string in CASTOR format for later processing.

#### Line 5+:

The format of lines 2-4 continues for each of the effects calculated. Line 11 then provides the information for the start of the second block or loading event, and the format repeats itself.

# Appendices

Appendix 1 – Traffic File Formats

### CASTOR File Format

In the table below, the Format column gives the storage type of the data.  IX refers to an integer of X number of digits, including leading or trailing zeros.

| Record | Unit | Format |
| --- | --- | --- |
| Head |  | I4 |
| Day |  | I2 |
| Month |  | I2 |
| Year |  | I2 |
| Hour |  | I2 |
| Minute |  | I2 |
| Second |  | I2 |
| Second/100 |  | I2 |
| Speed | dm/s | I3 |
| Gross Vehicle Weight - GVW | kg/100 | I4 |
| Length | dm | I3 |
| Number of Axles |  | I1 |
| Direction (1 or 2) |  | I1 |
| Lane |  | I1 |
| Transverse Location In Lane | dm | I3 |
| Weight Axle 1 | kg/100 | I3 |
| Spacing Axle 1 - Axle 2 | dm | I2 |
| Weight Axle 2 | kg/100 | I3 |
| Spacing Axle 2 - Axle 3 | dm | I2 |
| Spacing Axle 8 - Axle 9 | dm | I2 |
| Weight Axle 9 | kg/100 | I3 |

### BeDIT File Format

This file format is similar to CASTOR except that the maximum number of axles possible is 20, the axle spacings are given by a three digit number, and the direction is zero-based.

| Record | Unit | Format |
| --- | --- | --- |
| Head |  | I4 |
| Day |  | I2 |
| Month |  | I2 |
| Year |  | I2 |
| Hour |  | I2 |
| Minute |  | I2 |
| Second |  | I2 |
| Second/100 |  | I2 |
| Speed | dm/s | I3 |
| Gross Vehicle Weight - GVW | kg/100 | I4 |
| Length | dm | I3 |
| Number of Axles |  | I2 |
| Direction (zero-based) |  | I1 |
| Lane |  | I1 |
| Transverse Location In Lane | dm | I3 |
| Weight Axle 1 | kg/100 | I3 |
| Spacing Axle 1 - Axle 2 | dm | I3 |
| Weight Axle 2 | kg/100 | I3 |
| Spacing Axle 2 - Axle 3 | dm | I3 |
| Spacing Axle 19 - Axle 20 | dm | I3 |
| Weight Axle 20 | kg/100 | I3 |

### DITIS File Format

This file format is similar to BeDIT except that the wheel track width (i.e. transverse width of the vehicle, wheel-to-wheel) is included for each axle; the transverse positon units are cm; and the direction is one-based. This is for use with influence surfaces. Further, the year is a four-digit number.

| Field | Record | Unit | Format | Start |
| --- | --- | --- | --- | --- |
| 1 | Head |  | I4 | 1 |
| 2 | Day |  | I2 | 5 |
| 3 | Month |  | I2 | 7 |
| 4 | Year |  | I4 | 9 |
| 5 | Hour |  | I2 | 13 |
| 6 | Minute |  | I2 | 15 |
| 7 | Second |  | I2 | 17 |
| 8 | Second/100 |  | I2 | 19 |
| 9 | Speed | dm/s | I3 | 21 |
| 10 | Gross Vehicle Weight - GVW | kg/100 | I4 | 24 |
| 11 | Length | dm | I3 | 28 |
| 12 | Number of Axles |  | I2 | 31 |
| 13 | Direction (1 or 2) |  | I1 | 33 |
| 14 | Lane |  | I1 | 34 |
| 15 | Transverse Location In Lane | cm | I3 | 35 |
| 16 | Weight Axle 1 | kg/100 | I3 | 38 |
| 17 | Track Width Axle 1 | cm | I3 | 41 |
| 18 | Spacing Axle 1 - Axle 2 | dm | I3 | 44 |
| 19 | Weight Axle 2 | kg/100 | I3 | 47 |
| 20 | Track Width Axle 2 | cm | I3 | 50 |
| 21 | Spacing Axle 2 - Axle 3 | dm | I3 | 53 |
| 72 | Spacing Axle 19 - Axle 20 | dm | I3 | 206 |
| 73 | Weight Axle 20 | kg/100 | I3 | 209 |
| 74 | Track Width Axle 20 | cm | I3 | 212 |

### MON File Format

This file format is roughly similar to the BeDIT format but the units, order, and number of digits in each field varies to match common Australian WIM measurements. There is no defined maximum number of axles, but the last axle spacing is defined, allowing calculation (from the vehicle length) of the front and rear overhangs.

| Record | Unit | Format |
| --- | --- | --- |
| Head |  | I9 |
| Day |  | I2 |
| Month |  | I2 |
| Year |  | I4 |
| Hour |  | I2 |
| Minute |  | I2 |
| Second | ms | I5 |
| Number of axles |  | I2 |
| Number of axle groups |  | I2 |
| Gross Vehicle Weight - GVW | kg | I6 |
| Speed | km/h | I3 |
| Length | mm | I5 |
| Lane (1-based local) |  | I1 |
| Direction (0-based) |  | I1 |
| Transverse Location In Lane | mm | I4 |
| Weight Axle 1 | kg | I5 |
| Spacing Axle 1 - Axle 2 | mm | I5 |
| Weight Axle 2 | kg | I5 |
| Spacing Axle 2 - Axle 3 | mm | I5 |
| Weight Axle – Maximum defined | kg | I5 |
| Spacing Axle – Maximum defined | mm | I5 |

## Appendix 2 – References

- Caprani, C.C. (2005), Probabilistic Analysis of Highway Bridge Traffic Loading, PhD Thesis, School of Architecture, Landscape and Civil Engineering, University College Dublin, Ireland. http://www.colincaprani.com/research/phd-dissertation/

- Caprani, C.C. (2012), ‘Calibration of a congestion load model for highway bridges using traffic microsimulation’, Structural Engineering International, 22(3), August, in print.

- Caprani, C.C., OBrien, E.J. and McLachlan, G.J. (2008), ‘Characteristic traffic load effects from a mixture of loading events on short to medium span bridges’, Structural Safety, Vol. 30(5), September, pp. 394-404.

10.1016/j.strusafe.2006.11.006

- Grave, S.A.J. (2001), Modelling of Site-Specific Traffic Loading on Short to Medium Span Bridges, Ph.D. Thesis, Department of Civil Engineering, Trinity College Dublin.

- OBrien, E.J. and Caprani, C.C. (2005), ‘Headway modelling for traffic load assessment of short- to medium-span bridges’, The Structural Engineer, Vol 83, No. 16, August, pp. 33-36.

http://www.istructe.org/thestructuralengineer/HC/Abstract.asp?PID=5494